# Minimal discrete matches for target grain size distributions

Lorne Arnold

## Abstract

Soil is fundamentally a discrete material that is, nevertheless, commonly modeled as a continuum in part because of the computational expense of large-scale discrete element models (DEMs). Even at lab specimen scales, DEM’s computational cost may be substantial depending on the grain sizes being modeled. Despite these limitations, discrete models have proven useful in furthering our understanding of soil mechanics because they can spontaneously replicate realistic soil behavior as an emergent macro-scale property from a collection of particles following relatively simple interaction rules. This makes DEM an attractive tool for a multi-scale modeling approach where the constitutive behavior of a representative volume element (RV) is characterized with a discrete model and applied at a larger scale through a continuum model. The ability of the RV to represent a soil depends strongly on an appropriate grain size distribution match. However, in order to achieve computationally feasible models, even at lab scales, DEM simulations often use larger minimum particle sizes and more uniform distributions than their intended targets. Intuitively, discrete matches of different grain size distributions (GSDs) will require vastly different numbers of particles. But the precise relationship between GSD characteristics and the number of particles needed to match the distribution (and by extension the associated computational cost associated) is not intuitive. In this paper, we present the minimal discrete match (MDM) concept. The minimal discrete match is the smallest set of discrete particles needed to match a given GSD. We present a method for determining the MDM for any GSD and discuss strategies for finding the smallest MDM within a set of tolerances on the GSD. A mapping of USCS classification and MDM reveals a broad distribution of computational cost over several orders of magnitude for granular soils.

## Keywords

Discrete soil description, analytical solutions, open-source algorithms, computational cost

## Introduction

Soil is a fundamentally discrete granular material whose complex mechanical behavior emerges from the numerous interactions between its constituent parts. The discrete element method (DEM), introduced by Cundall and Strack (1979), has proven to be a powerful tool in exploring the inter-particle interactions of granular assemblies. With DEM, a vast parametric space exists where grain sizes, shapes, contact models, etc. can be systematically varied and their influences on macro-scale assembly behavior can be quantified.

These models pose several challenges that must be managed in order for their benefits to be realized. Due to the number of elements involved in typical DEM simulations, both the computational resources needed to run them and the ability to characterize and interpret them are non-trivial even at lab specimen scales. Over the years, the number of particles used in DEM simulations has increased, but not proportionally to the reduction in costs of computational resources. O’Sullivan (2014) showed that the order of magnitude of average number of particles in DEM simulations increased substantially from 1998 to 2014, from approx. O103 to O105, but fell far short of the O107 predicted by Cundall (2001) for “easy” geomechanics problems by 2011. More recent publications in DEM have shown that orders ranging from O106 (Sufian et al. (2021), Dong, Yan, and Cui (2022)) to O108 (Miyai et al. (2019), Fang et al. (2021), Zhang et al. (2024)) are possible, but rare and require high performance computing resources.

Because of the computational cost, DEM simulations whose particles approach realistic grain sizes generally model at the representative volume (RV) element scale. Several nuances to the RV concept exist, but broadly speaking, the RV represents the smallest volume of material to exhibit statistically consistent macro-scale behavior as its source material (Gitman, Askes, and Sluys 2007). In physical laboratory experiments and discrete numerical experiments, ensuring study samples are sufficiently large to behave as an RV is critical to the broader applicability of the results.

The RV size depends on several factors including the grain size distribution (GSD), sample density, loading conditions, grain shape and arrangement, and the behavior of interest. Studies have tied RV to sample height to maximum particle diameter ratios with values between 15 and 20 needed to achieve RV (Cantor and Ovalle 2025). Additionally, there is evidence that the average particle size (Zeraati-Shamsabadi and Sadrekarimi 2025) and overall polydispersity (Bandera et al. 2021) also influence RV size. Intuitively, with increasing GSD breadth, the number of particles needed in a DEM simulation of granular assemblies will also increase. Often, matching the true GSD of interest (e.g., from a physical soil sample) is sacrificed for computational efficiency by truncating or upscaling the distribution.

While the insights we gain from DEM simulations with upscaled or truncated GSDs are valuable, they come with acknowledged limitations on their broader applicability to natural soil fabrics. It becomes an important experimental design consideration, then, to understand the relative computational costs associated with different grain size distributions. For very specific GSDs, formulas can be used to calculate the number of particles needed. For example, Tyler and Wheatcraft (1992) provide such a formula for self-similar fractal distributions. However, they note that the formula results are sensitive to the choice of representative radius for a given range. Further, they concluded that a self-similar fractal distribution is a good model for pore structure, but not necessarily grain size. For general GSDs, particularly when a real soil is being modeled, an ideal approach would involve an analytical solution to the minimum number of particles required to reproduce the GSD.

This paper introduces such an approach through the minimal discrete match (MDM) concept. The MDM is the smallest set of discrete particles needed to match a given grain size distribution by mass. Note that the MDM is based on mass-size relationships only, not mechanical behavior. Therefore, it represents an important component to determining RV and relative computational cost, but it is not equivalent to an RV.

In order to characterize the MDM, descriptions of grain size distributions and granular samples are presented as mathematical sets and the rules describing several relationships between the sets are defined. Using these definitions, algorithms are presented for both an approximate and a rigorous solution for the MDM, which can be projected to calculate sample-scale particle quantities. The method is compared to published DEM models and shows a broad range of MDM magnitudes over several GSDs of interest in geotechnical engineering.

## Discrete definitions

Identifying the minimal discrete match requires rigorous mathematical definitions for particulate samples, S, and grain size distributions, G. Conceptually, each of these are collections of items (i.e., sets) with specific restrictions. The definitions are designed to capture the physical reality underlying a mechanical grain size analysis.

### Sample definition

A sample, S is an indexed set describing its particles as pairs of size, XS, and quantity, QS. XS is an ordered (i.e., each entry is larger than the previous) set of unique values of positive real numbers. Qs is an unordered set of positive integers related to XS by a shared indexing set for the sample, IS.

S={XS,QS∈IS}  1

where:

XS={xi:i∈IS} with x1<x2<⋯<xnS  2

QS={qi:i∈IS}  3

XS⊂R+ and QS⊂Z+  4

### Grain size distribution definition

A grain size distribution, G, has a similar structure. It is and indexed set describing size boundaries, XG, and masses, MG. Like XS, XG is an ordered set of unique values, however, it’s domain also includes zero (representing the pan in a typical sieve analysis). Like QS, MG shares an index (IG) with the size set, but it’s domain is not restricted to integers, only to non-negative real numbers. The values in MG represent the masses retained on a sieve with opening size XG.

G={XG,MG∈IG}  5

where:

XG={xj:j∈IG} where x1<x2<⋯<xnG  6

MG={mj:j∈IG}  7

XG⊂Rnon−neg ; MG⊂Rnon−neg  8

### Comparison definition

The relationship between S and G cad be defined in terms of how G describes S. In a later section, the topic of finding an instance of S that matches a target G will be addressed.

The relationship between S and G is formalized with the following conditions:

Condition 1: G is complete if and only if the final entry in MG is zero:

G is complete ⇔mnG=0  9

This condition ensures that xnG provides an upper bound to the sizes described by G. This is analogous to the limitation of scope in ASTM D6913 to grain sizes passing the 75-mm sieve, ensuring the cumulative GSD always reaches 100 percent at a known size (D18 Committee, n.d.).

Condition 2: G describes S (denoted G→S) if and only if Cond1 is met and the its smallest size is smaller than any size in S and its largest size is larger than any size in S:

G→S⇔ Cond1∧minXG<minXS∧maxXG≥maxXS  10

Condition 3: G describes S articulately if Cond2 is met and for all consecutive pairs of sizes in XS there exists at least one size in XG between them.

G→articulatelyS⇔Cond2∧∀xi,xi+1∈XS ∃ xj∈XG such that xi<xj≤xi+1  11

Condition 4: G describes S accurately if Cond2 is met and the combined masses of all the particles with sizes between every pair of sizes in XG is equal to the retained mass on the lower of the size pairs in G:

G→accuratelyS⇔ Cond2∧i∈ISxj<xi≤xj+1​qi⋅fxi=mj  12

where fx is a scaling function that converts size to mass.

Cond4 represents what is generally meant by a “match” between a sample and a grain size distribution. Note that Cond3 is not required for Cond4 to be satisfied.

Match error: The error for a match between G and S is simply a measure of how far Cond4 is from being met:

ErrorG,S=mj −i∈ISxj<xi≤xj+1​qi⋅fxi  13

### Physical interpretation

Some assumptions are required in order for S and G to be interpreted as a physical sample and sieve analysis (or a numerical version of the same). First, the sizes in XG are assumed to represent smallest enclosing diameter of whatever shapes the particles in S are. Second, as indicated in Equation 12, a size in XS is considered between two sizes in XG if it is larger than the xj and smaller than or equal to xj+1. In other words, particles pass through openings equal to their own size. Finally, the particles in S are assumed to be of similar enough shape that a single scaling function, fx, can represent the size-mass relationship for all of S. For spherical particles with of constant density (ρ), this is a straightforward definition:

fx=ρπ6x3  14

Note, however, that the use of this particular scaling function is not required in general. It is only necessary that fx be some mapping of x↦m. It would be possible to combine sets of differently shaped particles of different densities with respective mapping functions, but this paper will limit itself to assuming uniform, spherical particles.

## GSD study suite

This paper will analyze several grain size distributions in the range of interest in geotechnical engineering (although the methods are applicable over a wider range). Specifically a suite of 2750 randomized GSDs based on the sieve set specified in ASTM D6913 (14 opening sizes from 0.075 to 75 mm) are analyzed. The distributions were generated by adding random noise to normal mass distributions around a “central” size. The central size itself was randomly selected from the middle half of the sieves used. Between 5 and 14 consecutive sieves were used in each GSD. For classification purposes, articles smaller than 0.075 are assumed to be low plasticity silt, regardless of their size.

Table 1 summarizes the GSD suite using several traditional characteristics including coefficient of curvature, Cc, and coefficient of uniformity, Cu. The grain size index, IGS, as defined by Erguler (2016) and a new parameter, the curvature index, IC, are also presented. The curvature index is conceptually similar to the grain size index in that it is a ratio of areas related to the cumulative distribution function of the GSD. IC normalizes the area under the GSD curve to the trapezoidal area bounded by the percent passing the smallest size above the pan (xmin+1) and the largest size (xmax). Figure 1 shows a subset (about 20%) of the GSD suite and illustrates the definition of IC on an example GSD.

Table 1: Grain size distribution parameters in this study Parameter; Value(s) in study. Instances of G; 2750. Sieves in G; 5 to 14. xmin+1; 0.075 to 25 mm. xmax; 9.5 to 75 mm. Percent fines; 0.4% to 34%. Cc; 0.2 to 25.5. Cu; 1.2 to 214.5. IGS; 0.03 to 0.60. IC; 0.29 to 1.57.

Figure 1: A representative subset (~10%) of the GSD suite. The area ratio defining the curvature index is shown. The curve in the highlighted example has a curvature index of 1.09.

## Minimal discrete match solution

This section presents the use of the discrete definitions in the previous section in the formulation of the solution to the MDM problem. The solution takes the form of the MDM sample, SMDM, which contains the the minimum number of particles (NMDM) needed for a mass-based match to G. The solution is then projected to the simulation scale, allowing for an estimate of the number of particles in an simulation Nsim of arbitrary size.

### Mass-based matching

As a starting point, assuming that G is an articulate description of S limits the number of sizes in S to nG−1. Temporarily setting aside the minimization goal, any valid sizes may be selected for XS. From G, an indexed set of mass ratios, Φ, can be defined to describe the masses of each size range relative to the mass retained on the second to largest sieve (recall that no mass is retained on the largest sieve):

Φ=mjmnG−1:j∈IG−1; with elements ϕj  15

The length of Φ is one less than IG, making it equal in length to IS because of Cond3.

Similarly, a set of volume ratios, Z can be defined to describe the volume per particle relative to the largest particle:

Z=fxnSfxi:i∈IS; with elements ζi  16

Note that Z is referred to as a volume ratio to avoid confusion with Φ despite the fact that the mapping function fx converts size to mass. Interpreting Z as a volume ratio is valid under a constant density assumption.

Because they are ratios relative to the largest considered size, ϕnS=ζnS=1. Since Φ describes relative masses of each size and Z describes the relative volume (and mass) per particle of each size, their product will describe a relative quantity ratio, K:

K=ϕi×ζi:i∈IS; with elements κi  17

The quantity ratio K is the relative number of particles of each of the assumed sizes in XS needed to satisfy Cond4. It is directly proportional to the key target parameter, QS. Being the product of ϕnS and ζnS, κnS will be an integer equal to 1, making it the smallest allowable candidate for qnS. Unfortunately, with the exception of κnS, the entries in K are not guaranteed to be integers. Rounding each element in K to the nearest integer provides a first-order, approximate solution for the minimal discrete match.

Sapprox={X,intK∈IS}≈Smin  18

Depending on the application and acceptable error tolerance, Sapprox may be a suitable substitute for Smin. The error for the approximate solution that assumed fixed sizes can be reduced using the following fixed size (FS) algorithm:

- Find K using Equation 17 and define an integer value i=0.

- Use inti+1×K to approximate QSi and build Si={XS,QSi}.

- Calculate the error, Ei, between G and Si with Equation 13.

- If Ei exceeds the desired tolerance, increase i by 1.

- Repeat Steps FS2 through FS4 until the desired tolerance is reached.

This approach can reduce the match error to an arbitrarily low value, but the total number of particles grows by a factor of QS0 with each iteration.

But since the sizes in XS are only bounded by XG and not explicitly defined, the opportunity exists to find a better selection of entries in XS that will minimize QS. The best allowable sizes for XS can be found using the following spanned integer (SI) algorithm:

- Find the quantity ratios associated with the minimum (−) and maximum (+) allowable sizes in x1 through xnS−1 (i.e. K− and K+) and define an integer value i=0.

- Check whether an integer is spanned between each entry in K− and K+. An integer is spanned between two consecutive numbers a and b if ⌈a⌉≤⌊b⌋ (i.e., a is rounded to the next highest integer and b is rounded to the next lowest integer).

- If a spanned integer (SI) does not exist for each entry, repeat Steps SI1 and SI2 with incrementally larger sizes for xnS.

- If, after reaching the maximum allowable particle size for xnS, an SI does not exist between each entry in K− and K+, increase i by 1 and perform Step FS2 (previous algorithm) and repeat Steps SI1 to SI3 (this algorithm).

- When an integer quantity exists between each entry in K− and K+, these integers can be used to populate the quantity set QSI.

- Invert the mass scaling function fx on the ratio Φ / QSI to identify the compatible sizes for the spanned integer quantities:

XSI=f−1ΦQSI  19

Because QSI was generated using the allowable sizes of XS and the mass ratios of the target G, the resulting XSI satisfies Cond4 with QSI as the smallest possible quantity of particles, or the minimal discrete match:

MDM=Smin={XSI,QSI}  20

The total number of particles in the MDM is the sum of the quantities in QSI:

NMDM=i∈IS​qSI,i  21

The effectiveness of the spanned integer algorithm is shown in the rapid convergence to numeric zero error compared to a fixed-size approximation in Figure 2. The figure shows both approaches attempting to find a suitable sample for each of the GSDs shown in Figure 1 with an allowable total error tolerance of 1 percent. In nearly all cases (over 98% for the GSDs in this study), the spanned integer approach requires no quantity-increasing iteration (i.e., Step SI4 is skipped). Where iteration is needed, it converges to numerical zero within one or two iterations. The fixed size approach converges with between 1 and 15 iterations and retains measurable error. This difference is significant, not because the spanned integer algorithm itself is computationally expensive, but because each iteration represents an increase in total particles in the final discrete sample.

Figure 2: Packing Algorithm Convergence

In some sense, the FS approach would appear invalid since it tends to find larger “minimal” solutions than the SI approach, but there are scenarios in which it can be useful. For example, in cases where specific particle sizes (as opposed to ranges) are needed, as is the case with space-filling problems (Botet, Kwok, and Cabane 2021). In the context of mass-distribution fitting, the FS results show how sensitive the minimal discrete matching problem is to the selection of sizes. The SI algorithm finds the ideal sizes where the relative particle quantities fall neatly into place to match the target GSD. Even over the fairly small available size ranges, the FS algorithm performance shows that deviation from these ideal sizes can carry a significant penalty. The results in the rest of this paper use the spanned integer algorithm.

### Projection to simulation scale

The volume of the MDM is not the same as the RV or any specific physical sample size. But converting the MDM results to a simulation scale is straightforward if the volume and target void ratio are known. The number of particles in a simulation-scale sample (Nsim) can be estimated using:

Nsim=NMDM×VsimVMDM×1+e  22

VMDM is the volume of solids in the MDM, which can be calculated from the total MDM mass (Equation 12) and particle density. The result of Equation 22 should be considered an estimate of the total particles of the full-scale DEM as it does not take into account boundary effects or the particle-generating algorithms used by DEM codes. Also note that whether the simulation volume, Vsim, meets or is intended to meet the criteria for RV is the modeler’s responsibility and outside the scope of this work.

## Results

This section presents the results of a comparison between the MDM methods prediction of Nsim and an independent DEM study as well as the results for NMDM for the GSD suite described in Table 1.

### Comparison to independent DEM data

The MDM-based simulation-scale estimate method was tested against the DEM models from a study on upscaled GSDs of Athabasca oil tailings sand by Zeraati-Shamsabadi and Sadrekarimi (2025). The study upscaled the actual sand GSD (for computational efficiency) with a scale factor (SF) ranging from 5 to 20 and produced simulation samples with diameters ranging from 50 to 175 mm with consolidated sample heights and void ratios from 23.6 to 26.0 mm and 0.64 to 0.8, respectively. According to the USCS, unscaled Athabasca sand and the GSDs up to SF of 15 are classified as “poorly graded sand”; the coarsest model (SF=20) is “poorly graded sand with gravel.”

Figure 3 shows a strong agreement between of the total number of simulation particles predicted by the MDM method and those reported in the Zeraati-Shamsabadi and Sadrekarimi (2025) study. Relative to the predicted values, the study reported a moderately larger number of particles for all the samples. This suggests the DEM models used different discrete representation of the scaled GSDs than the MDM, which is expected. Even so, the consistent trend confirms the MDM captures the relative computational efforts and provides a good estimate of the actual Nsim independent of the specific discrete GSD match. The figure also shows one of the most useful aspects of the MDM method: predicting computational effort needed for extremely high resolution DEM models without generating them. Relative to the highest resolution models in the study (SF=5), the MDM method indicates simulations of unscaled Athabasca sand would require roughly 102 times the number of particles.

Figure 3: Comparison of MDM-predicted sample sizes for scaled GSDs in Z study. Predictions for unscaled models of A sand.

### MDMs in the GSD suite

The minimal discrete matches for the GSD suite described in Table 1 range in magnitude from O101 to O1010 particles. As intuition suggests, increasing the percent mass of the smallest particle sizes or the ratio of the largest to the smallest particle sizes both tend to increase NMDM. Figure 4 illustrates a scattered linear relationship between ϕ1 (subscript 1 indicates the smallest particle size) and NMDM in log-log space for constant volume ratio, ζ1. For ϕ1≈0.4, log10ζ1 correlates roughly 1:1 with NMDM. The color bands of log10ζ1 indicate distinct combinations of min and max sieve sizes in the target G.

Figure 4: Total particles in MDM with mass ratio. Color scale indicates volume ratio.

A similar pattern exists between NMDM , ζ1, and IC as shown in Figure 5. At a given volume ratio, an increase in curvature index tends to result in an increased NMDM. For IC≈0.7, log10ζ1 provides a very rough 1:1 correlation with NMDM. Curves with high IC tend to involve larger relative masses at lower sizes than those with low IC.

Figure 5: Total particles in MDM with curvature index. Color scale indicates volume ratio.

The grain size parameters Cc, Cu, and IGS were found to not correlate well with NMDM. However, IGS is helpful for visualizing the breadth of correlation between NMDM and USCS classifications. Figure 6 shows the range of NMDM associated with the granular USCS classifications in this study. The lower boundary of the data results indicate that all but the broadest distributions have instances that can be matched with NMDM magnitudes of O102 or lower. Yet even the classifications with the smallest observed matches also have numerous instances requiring magnitudes from O106 to O109.

Figure 6: Total particles in MDM with USCS classification.

Relative to “poorly graded gravel”, “well-graded gravel” tends to have larger NMDM. Where “with sand” is applicable to the gravel group name, this tendancy and lower limit of NMDM remains the same, but the upper range of NMDM is higher. Instances where “with silt and sand” are applicable to the gravel group name were rare in the generated GSDs and had very high values of NMDM. The rarity and large NMDM for these gradations are due to strict requirements for these group names, which only a small range of values across a broad range of particle sizes can satisfy.

Relative to “poorly graded sand”, “well-graded sand” tends to have a similar (and quite broad) range of NMDM. The addition of “with gravel” or “with silt” makes very little difference in the range of NMDM at similar ranges of IGS (although a small increase in lower bound is apparent in “well-graded sand with gravel”. Even “silty sand” appears to share orders of magnitude with “poorly graded sand” at comparable ranges of IGS. Similar to the trends for gravels, sands whose group names include both silt and gravel were rare in the generated GSDs and had high values of NMDM.

## Discussion

The SI algorithm component of the approach presented here is significantly more effective than the FS algorithm in finding the MDM. However, whether this difference is critical in most geomechanics applications is unclear. Intuitively, MDM solutions for distributions with smallest quantities of particles will have the most sensitivity to errors and seem to benefit from the SI algorithm. On the other hand, these distributions are likely to have the smallest volumes relative to their representative volumes and will therefore have Nsim≫NMDM, which will tend to reduce the errors associated with the FS algorithm substantially. Additionally, as it allows greater control over particle sizes, an approximate solution from the FS algorithm may even be preferable depending on the application and the desired rigor in the S:G match.

Both of the MDM approaches assume a single particle size between each adjacent sieve pair, but natural soil grain distributions are known to be broadly distributed between sieves (Ghalib and Hryciw 1999). This aspect of MDM solution stems from the use of Cond3, which is important in finding the solution, but can be abandoned after the solution is found in most cases. Except where the solution contains only a single particle at a given size, the possibility exists for replacing the single size and associated quantity with an equal number of particles with varying (but allowable) sizes. The greater the number of particles at a given size, the more potential there is to represent them with a distribution of sizes.

The comparison between predicted and reported Nsim values in Figure 3 show the value of the MDM concept for experimental design. As mentioned previously, differences between the reported and predicted values suggest the DEM models used a different discrete representation of the scaled GSDs than the MDM. The fact that the reported values are consistently, though modestly, higher than the MDM predictions is expected since the MDM is defined to be a lower bound of possible values. It is worth noting that the MDM concept is silent as to whether any valid discrete representation of a GSD is “better” than another, only which has fewest particles. From a mechanics perspective, non-minimal GSD representations may have advantages that supersede increases in Nsim; an idea worth further study.

The MDM concept makes much of providing an accurate representation of grain size distribution because mechanical behavior cannot be fully decoupled from GSD. However, when it is used in experimental design and to explore feasibility, it should be paired with the understanding that partial decoupling of mechanical behavior from GSD is sometimes possible. For example, studies have shown that for certain combinations of GSD and density, there may be significant portions of the soil mass that are not mechanically engaged with the soil matrix (Sufian et al. 2021). This may lead to thresholds of particle size that can be omitted from a given simulation without significant impact on the behavior being studied. In such cases, the MDM concept is still valuable for comparing relative computational effort with varying levels of truncation.

## Conclusions

This paper described algorithms to 1) find a first-order approximation of a minimal discrete match (MDM) to general grain size distributions, 2) quantify the error in approximate minimal matches, and 3) find a rigorous solution for the MDM with a spanned integer approach. The following conclusions are presented as the key out comes of the study on the MDM algorithms, comparison of predictions with independent DEM data, and a study on an array of GSDS:

- The MDM is a valuable tool in quantifying relative number of particles needed to create DEM models of different grain size distributions. This can provide important insight into computational cost and feasibility during experimental design. To be used effectively, it must be accompanied by an understanding of the representative volume concept.

- The MDM results for a large distribution of USCS show several trends: increasing the ratio of maximum to minimum particle size, the relative mass of the smallest particles, and the curvature index all increase the size of the MDM.

- The distribution of MDM size across USCS classifications indicates that a wide range of realistic granular soils are feasible for DEM modeling. The distribution also indicates that neither USCS classification, curvature index, size, and mass ratios alone are sufficient to characterize MDM size. This indicates that MDM magnitudes for specific GSDs of interest should calculated rather than estimated based on index parameters.

## Data and code availability

The Python code used to generate data, solve for minimal discrete matches, and plot figures are available on GitHub.

Bandera, Sara, Catherine O’Sullivan, Paul Tangney, and Stefano Angioletti-Uberti. 2021. “Coarse-Grained Molecular Dynamics Simulations of Clay Compression.” Computers and Geotechnics 138 (October): 104333. https://doi.org/10.1016/j.compgeo.2021.104333.

Botet, Robert, Sylvie Kwok, and Bernard Cabane. 2021. “Filling Space with Polydisperse Spheres in a Non-Apollonian Way.” Journal of Physics A: Mathematical and Theoretical 54 (19): 195201. https://doi.org/10.1088/1751-8121/abef81.

Cantor, David, and Carlos Ovalle. 2025. “Sample Size Effects on the Critical State Shear Strength of Granular Materials with Varied Gradation and the Role of Column-Like Local Structures.” Géotechnique 75 (1): 29–40. https://doi.org/10.1680/jgeot.23.00032.

Cundall, P. A. 2001. “A Discontinuous Future for Numerical Modelling in Geomechanics?” Proceedings of the Institution of Civil Engineers - Geotechnical Engineering 149 (1): 41–47. https://doi.org/10.1680/geng.2001.149.1.41.

Cundall, P. A., and O. D. L. Strack. 1979. “A Discrete Numerical Model for Granular Assemblies.” Géotechnique 29 (1): 47–65. https://doi.org/10.1680/geot.1979.29.1.47.

D18 Committee. n.d. “Test Methods for Particle-Size Distribution (Gradation) of Soils Using Sieve Analysis.” https://doi.org/10.1520/d6913_d6913m-17.

Dong, Youkou, Dingtao Yan, and Lan Cui. 2022. “An Efficient Parallel Framework for the Discrete Element Method Using GPU.” Applied Sciences 12 (6): 3107. https://doi.org/10.3390/app12063107.

Erguler, Zeynal Abiddin. 2016. “A Quantitative Method of Describing Grain Size Distribution of Soils and Some Examples for Its Applications.” Bulletin of Engineering Geology and the Environment 75 (2): 807–19. https://doi.org/10.1007/s10064-015-0790-1.

Fang, Luning, Ruochun Zhang, Colin Vanden Heuvel, Radu Serban, and Dan Negrut. 2021. “Chrono::GPU: An Open-Source Simulation Package for Granular Dynamics Using the Discrete Element Method.” Processes 9 (10): 1813. https://doi.org/10.3390/pr9101813.

Ghalib, Ali M., and Roman D. Hryciw. 1999. “Soil Particle Size Distribution by Mosaic Imaging and Watershed Analysis.” Journal of Computing in Civil Engineering 13 (2): 80–87. https://doi.org/10.1061/(ASCE)0887-3801(1999)13:2(80).

Gitman, I. M., H. Askes, and L. J. Sluys. 2007. “Representative Volume: Existence and Size Determination.” Engineering Fracture Mechanics 74 (16): 2518–34. https://doi.org/10.1016/j.engfracmech.2006.12.021.

Miyai, Shinichiro, Murino Kobayakawa, Takuya Tsuji, and Toshitsugu Tanaka. 2019. “Influence of Particle Size on Vertical Plate Penetration into Dense Cohesionless Granular Materials (Large-Scale DEM Simulation Using Real Particle Size).” Granular Matter 21 (4): 105. https://doi.org/10.1007/s10035-019-0961-z.

O’Sullivan, C. 2014. “Advancing Geomechanics Using DEM.” In, edited by Kenichi Soga, Krishna Kumar, Giovanna Biscontin, and Matthew Kuo, 21–32. CRC Press. http://www.crcnetbase.com/doi/abs/10.1201/b17395-4.

Sufian, Adnan, Marion Artigaut, Thomas Shire, and Catherine O’Sullivan. 2021. “Influence of Fabric on Stress Distribution in Gap-Graded Soil.” Journal of Geotechnical and Geoenvironmental Engineering 147 (5): 04021016. https://doi.org/10.1061/(ASCE)GT.1943-5606.0002487.

Tyler, Scott W., and Stephen W. Wheatcraft. 1992. “Fractal Scaling of Soil Particle-Size Distributions: Analysis and Limitations.” Soil Science Society of America Journal 56 (2): 362–69. https://doi.org/10.2136/sssaj1992.03615995005600020005x.

Zeraati-Shamsabadi, Mohammad, and Abouzar Sadrekarimi. 2025. “A DEM Study on the Effects of Specimen and Particle Sizes on Direct Simple Shear Tests.” Granular Matter 27 (2). https://doi.org/10.1007/s10035-025-01513-y.

Zhang, Ruochun, Bonaventura Tagliafierro, Colin Vanden Heuvel, Shlok Sabarwal, Luning Bakke, Yulong Yue, Xin Wei, Radu Serban, and Dan Negruţ. 2024. “Chrono DEM-Engine: A Discrete Element Method Dual-GPU Simulator with Customizable Contact Forces and Element Shape.” Computer Physics Communications 300 (July): 109196. https://doi.org/10.1016/j.cpc.2024.109196.